Secure Tunneling and Endpoint Configuration

TechVista

Tunnel Endpoint Router and Public Interface Config

| Tunnel ID | Source Router | Destination Router | Source Interface / IP Address | Destination IP Address |
| --- | --- | --- | --- | --- |
| 11011101 | CORE-RTR-T_VISTA-1 | CORE-RTR-C_PORT-1 | G0/0/0 110.110.110.1 | 111.111.111.1 |
| 11011201 | CORE-RTR-T_VISTA-1 | CORE-RTR-N_CYBERIA-1 | G0/0/0 110.110.110.1 | 112.112.112.1 |
| 11011102 | CORE-RTR-T_VISTA-2 | CORE-RTR-C_PORT-2 | G0/0/0 110.110.110.2 | 111.111.111.2 |
| 11011202 | CORE-RTR-T_VISTA-2 | CORE-RTR-N_CYBERIA-2 | G0/0/0 110.110.110.2 | 112.112.112.2 |

Tunnel ID and Subnet Assignment

| Tunnel ID | IP Subnet | Local Tunnel Endpoint | Remote Tunnel Endpoint |
| --- | --- | --- | --- |
| 11011101 | 192.168.1.0/30 | 192.168.1.1 | 192.168.1.2 |
| 11011201 | 192.168.1.4/30 | 192.168.1.5 | 192.168.1.6 |
| 11011102 | 192.168.1.8/30 | 192.168.1.9 | 192.168.1.10 |
| 11011202 | 192.168.1.12/30 | 192.168.1.13 | 192.168.1.14 |

CodePort

Tunnel Endpoint Router and Public Interface Config

| Tunnel ID | Source Router | Destination Router | Source Interface / IP Address | Destination IP Address |
| --- | --- | --- | --- | --- |
| 11111001 | CORE-RTR-C_PORT-1 | CORE-RTR-T_VISTA-1 | G0/0/0 111.111.111.1 | 110.110.110.1 |
| 11111201 | CORE-RTR-C_PORT-1 | CORE-RTR-N_CYBERIA-1 | G0/0/0 111.111.111.1 | 112.112.112.1 |
| 11111002 | CORE-RTR-C_PORT-2 | CORE-RTR-T_VISTA-2 | G0/0/0 111.111.111.2 | 110.110.110.2 |
| 11111202 | CORE-RTR-C_PORT-2 | CORE-RTR-N_CYBERIA-2 | G0/0/0 111.111.111.2 | 112.112.112.2 |

Tunnel ID and Subnet Assignment

| Tunnel ID | IP Subnet | Local Tunnel Endpoint | Remote Tunnel Endpoint |
| --- | --- | --- | --- |
| 11111001 | 192.168.1.0/30 | 192.168.1.2 | 192.168.1.1 |
| 11111201 | 192.168.1.16/30 | 192.168.1.17 | 192.168.1.18 |
| 11111002 | 192.168.1.8/30 | 192.168.1.10 | 192.168.1.9 |
| 11111202 | 192.168.1.20/30 | 192.168.1.21 | 192.168.1.22 |

NeoCyberia

Tunnel Endpoint Router and Public Interface Config

| Tunnel ID | Source Router | Destination Router | Source Interface / IP Address | Destination IP Address |
| --- | --- | --- | --- | --- |
| 11211001 | CORE-RTR-N_CYBERIA-1 | CORE-RTR-T_VISTA-1 | G0/0/0 112.112.112.1 | 110.110.110.1 |
| 11211101 | CORE-RTR-N_CYBERIA-1 | CORE-RTR-C_PORT-1 | G0/0/0 112.112.112.1 | 111.111.111.1 |
| 11211002 | CORE-RTR-N_CYBERIA-2 | CORE-RTR-T_VISTA-2 | G0/0/0 112.112.112.2 | 110.110.110.2 |
| 11211102 | CORE-RTR-N_CYBERIA-2 | CORE-RTR-C_PORT-2 | G0/0/0 112.112.112.2 | 111.111.111.2 |

Tunnel ID and Subnet Assignment

| Tunnel ID | IP Subnet | Local Tunnel Endpoint | Remote Tunnel Endpoint |
| --- | --- | --- | --- |
| 11211001 | 192.168.1.4/30 | 192.168.1.6 | 192.168.1.5 |
| 11211101 | 192.168.1.16/30 | 192.168.1.18 | 192.168.1.17 |
| 11211002 | 192.168.1.12/30 | 192.168.1.14 | 192.168.1.13 |
| 11211102 | 192.168.1.20/30 | 192.168.1.22 | 192.168.1.21 |

CORE-RTR-T_VISTA-1:

license boot module c2900 technology-package securityk9

y

do write

do reload

y

crypto isakmp policy 10

authentication pre-share

hash md5

encryption aes 256

ex

crypto isakmp key MentoraKey address 111.111.111.1

crypto isakmp key MentoraKey address 112.112.112.1

crypto ipsec transform-set [T_V1=>C_P1,N_C1]_SET esp-aes 256 esp-sha-hmac

access-list 111 permit ip 10.60.110.0 255.255.255.0 10.60.111.0 255.255.255.0

access-list 112 permit ip 10.60.110.0 255.255.255.0 10.60.112.0 255.255.255.0

crypto map  [T_V1=>C_P1,N_C1]_MAP 10 ipsec-isakmp

set peer 111.111.111.1

set transform-set [T_V1=>C_P1,N_C1]_SET

match address 111

set security-association lifetime seconds 86400

ex

crypto map  [T_V1=>C_P1,N_C1]_MAP 20 ipsec-isakmp

set peer 112.112.112.1

set transform-set [T_V1=>C_P1,N_C1]_SET

match address 112

set security-association lifetime seconds 86400

ex

interface GigabitEthernet0/0/0

crypto map [T_V1=>C_P1,N_C1]_MAP

do write

CORE-RTR-T_VISTA-2:

license boot module c2900 technology-package securityk9

y

do write

do reload

y

crypto isakmp policy 10

authentication pre-share

hash md5

encryption aes 256

ex

crypto isakmp key MentoraKey address 111.111.111.2

crypto isakmp key MentoraKey address 112.112.112.2

crypto ipsec transform-set [T_V2=>C_P2,N_C2]_SET esp-aes 256 esp-sha-hmac

access-list 111 permit ip 10.60.110.0 255.255.255.0 10.60.111.0 255.255.255.0

access-list 112 permit ip 10.60.110.0 255.255.255.0 10.60.112.0 255.255.255.0

crypto map  [T_V2=>C_P2,N_C2]_MAP 10 ipsec-isakmp

set peer 111.111.111.2

set transform-set [T_V2=>C_P2,N_C2]_SET

match address 111

set security-association lifetime seconds 86400

ex

crypto map  [T_V2=>C_P2,N_C2]_MAP 20 ipsec-isakmp

set peer 112.112.112.2

set transform-set [T_V2=>C_P2,N_C2]_SET

match address 112

set security-association lifetime seconds 86400

ex

interface GigabitEthernet0/0/0

crypto map [T_V2=>C_P2,N_C2]_MAP

do write

CORE-RTR-C_PORT-1:

license boot module c2900 technology-package securityk9

y

do write

do reload

y

crypto isakmp policy 10

authentication pre-share

hash md5

encryption aes 256

ex

crypto isakmp key MentoraKey address 110.110.110.1

crypto isakmp key MentoraKey address 112.112.112.1

crypto ipsec transform-set [C_P1=>T_V1,N_C1]_SET esp-aes 256 esp-sha-hmac

access-list 110 permit ip 10.60.111.0 255.255.255.0 10.60.110.0 255.255.255.0

access-list 112 permit ip 10.60.111.0 255.255.255.0 10.60.112.0 255.255.255.0

crypto map  [C_P1=>T_V1,N_C1]_MAP 10 ipsec-isakmp

set peer 110.110.110.1

set transform-set [C_P1=>T_V1,N_C1]_SET

match address 110

set security-association lifetime seconds 86400

ex

crypto map  [C_P1=>T_V1,N_C1]_MAP 20 ipsec-isakmp

set peer 112.112.112.1

set transform-set [C_P1=>T_V1,N_C1]_SET

match address 112

set security-association lifetime seconds 86400

ex

interface GigabitEthernet0/0/0

crypto map  [C_P1=>T_V1,N_C1]_MAP

do write

CORE-RTR-C_PORT-2:

license boot module c2900 technology-package securityk9

y

do write

do reload

y

crypto isakmp policy 10

authentication pre-share

hash md5

encryption aes 256

ex

crypto isakmp key MentoraKey address 110.110.110.2

crypto isakmp key MentoraKey address 112.112.112.2

crypto ipsec transform-set [C_P2=>T_V2,N_C2]_SET esp-aes 256 esp-sha-hmac

access-list 110 permit ip 10.60.111.0 255.255.255.0 10.60.110.0 255.255.255.0

access-list 112 permit ip 10.60.111.0 255.255.255.0 10.60.112.0 255.255.255.0

crypto map  [C_P2=>T_V2,N_C2]_MAP 10 ipsec-isakmp

set peer 110.110.110.2

set transform-set [C_P2=>T_V2,N_C2]_SET

match address 110

set security-association lifetime seconds 86400

ex

crypto map  [C_P2=>T_V2,N_C2]_MAP 20 ipsec-isakmp

set peer 112.112.112.2

set transform-set [C_P2=>T_V2,N_C2]_SET

match address 112

set security-association lifetime seconds 86400

ex

interface GigabitEthernet0/0/0

crypto map  [C_P2=>T_V2,N_C2]_MAP

do write

CORE-RTR-N_CYBERIA-1:

license boot module c2900 technology-package securityk9

y

do write

do reload

y

crypto isakmp policy 10

authentication pre-share

hash md5

encryption aes 256

ex

crypto isakmp key MentoraKey address 110.110.110.1

crypto isakmp key MentoraKey address 111.111.111.1

crypto ipsec transform-set [N_C1=>T_V1,C_P1]_SET esp-aes 256 esp-sha-hmac

access-list 110 permit ip 10.60.112.0 255.255.255.0 10.60.110.0 255.255.255.0

access-list 111 permit ip 10.60.112.0 255.255.255.0 10.60.111.0 255.255.255.0

crypto map  [N_C1=>T_V1,C_P1]_MAP 10 ipsec-isakmp

set peer 110.110.110.1

set transform-set [N_C1=>T_V1,C_P1]_SET

match address 110

set security-association lifetime seconds 86400

ex

crypto map  [N_C1=>T_V1,C_P1]_MAP 20 ipsec-isakmp

set peer 111.111.111.1

set transform-set [N_C1=>T_V1,C_P1]_SET

match address 111

set security-association lifetime seconds 86400

ex

interface GigabitEthernet0/0/0

crypto map  [N_C1=>T_V1,C_P1]_MAP

do write

CORE-RTR-N_CYBERIA-2:

license boot module c2900 technology-package securityk9

y

do write

do reload

y

crypto isakmp policy 10

authentication pre-share

hash md5

encryption aes 256

ex

crypto isakmp key MentoraKey address 110.110.110.2

crypto isakmp key MentoraKey address 111.111.111.2

crypto ipsec transform-set [N_C2=>T_V2,C_P2]_SET esp-aes 256 esp-sha-hmac

access-list 110 permit ip 10.60.112.0 255.255.255.0 10.60.110.0 255.255.255.0

access-list 111 permit ip 10.60.112.0 255.255.255.0 10.60.111.0 255.255.255.0

crypto map  [N_C2=>T_V2,C_P2]_MAP 10 ipsec-isakmp

set peer 110.110.110.2

set transform-set [N_C2=>T_V2,C_P2]_SET

match address 110

set security-association lifetime seconds 86400

ex

crypto map  [N_C2=>T_V2,C_P2]_MAP 20 ipsec-isakmp

set peer 111.111.111.2

set transform-set [N_C2=>T_V2,C_P2]_SET

match address 111

set security-association lifetime seconds 86400

ex

interface GigabitEthernet0/0/0

crypto map  [N_C2=>T_V2,C_P2]_MAP

do write