Home-RTR

ip route 0.0.0.0 0.0.0.0 1.1.1.2

RTR-ISP-Partner

ip route 192.168.144.0 255.255.255.0 1.1.1.1

ip route 10.60.111.0 255.255.255.0 2.2.2.2

CORE-RTR-C_PORT-1

ip route 192.168.144.0 255.255.255.0 2.2.2.1

ip local pool VPNPOOL 10.60.111.220 10.60.111.230

aaa new-model

aaa authentication login USERVPN local

aaa authorization network GROUPVPN local

username uservpn secret CiscoVPN

crypto isakmp policy 100

encryption aes 256

hash sha

authentication pre-share

group 5

lifetime 3600

exit

crypto isakmp client configuration group GROUPVPN

key CiscoGroupVPN

pool VPNPOOL

crypto ipsec transform-set SETVPN esp-aes esp-sha-hmac

crypto dynamic-map DYNMAP 100

set transform-set SETVPN

reverse-route

crypto map STATICMAP client authentication list USERVPN

crypto map STATICMAP isakmp authorization list GROUPVPN

crypto map STATICMAP client configuration address respond

crypto map STATICMAP 10 ipsec-isakmp dynamic DYNMAP

interface g0/1/0

crypto map STATICMAP

Host: 2.2.2.2

Group Name: GROUPVPN

Group Password: CiscoGroupVPN

Username: uservpn

Password: CiscoVPN