| Requirement | Solution | Packet Tracer Instructions |
| --- | --- | --- |
| Manage company emails and enable email exchange between employees. | Set up POP3 and SMTP servers. | Add a server in Packet Tracer. Configure the email service with POP3 and SMTP settings. Assign proper IP addresses. |
| Provide internal storage for company files. | Establish FTP servers for secure storage and retrieval of internal organizational data. | Set up a server in Packet Tracer. Enable the FTP service. Set up usernames and passwords for access. |
| Enable employee access to client management applications and data through user-friendly domain names instead of IP addresses. | Set up internal WEB servers and implement a DNS server to translate IP addresses into domain names. | In Packet Tracer, configure a server with HTTP and DNS services. Create DNS records mapping domain names to server IPs. |
| Ensure fast communication between company branches. | Use Metro Ethernet infrastructure with fiber-optic connectivity provided by the ISP. | In Packet Tracer, use switches and routers to create VLANs. Simulate a high-speed link using fiber-optic connections between devices. |
| Maintain smooth, fast, and load-free internal network traffic. | Design an optimized network structure, implement VLANs for different departments, and divide into subnets. | Create VLANs in Packet Tracer. Assign devices to appropriate VLANs and configure inter-VLAN routing. |
| Allow the network administrator to manage the network remotely. | Enable remote access (VTY) to routers and switches. | Enable VTY and SSH on Packet Tracer routers and switches. Simulate remote access using a PC with SSH client software. |
| Provide reliable, uninterrupted internet connectivity for the company. | Lease a public IP address from the ISP and implement NAT to translate private addresses to public ones. | Set up NAT on Packet Tracer routers. Use private IPs for internal devices and a public IP for external communication. |
| Provide wireless internet access for employees and visitors. | Implement wireless access points (WAPs) for secure connectivity via mobile phones, tablets, and laptops. | Add a wireless router in Packet Tracer. Configure SSIDs and enable secure wireless encryption. |
| Enable selective internet access for certain employees. | Connect to an ISP and implement NAT on the outbound router. | Use Packet Tracer ACL configurations to restrict internet access. Combine with NAT settings on the router. |